# ANNEX IV : Budget breakdown (Model financial offer)

Nota Bene: All amounts are to be quoted excluding taxes. VAT, if applicable, is to be mentioned separately. Please refer to tender documents, notably the Instructions to tenderers, section 11 Content of tenders.

# Page No 1 [of…]

# PUBLICATION REFERENCE: <reference number>	NAME OF TENDERER: <name>

| A |  |  |  |  |  |  |  | C |  | D |  | E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item number |  | DESCRIPTION |  | UNIT |  | Quantity |  | specifications offered (incl brand/model) |  | Unit costs with delivery DDP RESEN EUR |  | total EUR |  |
| 1 |  | Half-cut monocrystalline photovoltaic modules with a power output of 500 Wp Dimensions of 1,909 × 1,134 × 30 mm |  | piece |  | 40 |  |  |  |  |  |  |  |
| 2 |  | Aluminum profiles as a structural framework, including all necessary connecting elements, with a length of 50 cm or similar |  | piece |  | 98 |  |  |  |  |  |  |  |
| 3 |  | Aluminum end clamps for securing the modules |  | piece |  | 36 |  |  |  |  |  |  |  |
| 4 |  | Aluminum mid clamps for securing the modules |  | piece |  | 62 |  |  |  |  |  |  |  |
| 5 |  | Three-phase inverter with a nominal power output of 17.50 kW, equipped with integrated DC and AC surge protection devices and an integrated plant monitoring system. Reference type: Fronius Symo Advanced 17.5-3-M or equivalent Warranty 5 years |  | piece |  | 1 |  |  |  |  |  |  |  |
| 6 |  | AC cabinet with • dimensions of 400 × 500 × 200 mm, • minimum mechanical protection rating of IP65, • impact resistance IK08, • Protection Class II, • halogen-free construction, • UV- and weather-resistant eclosure, • Suitable for a rated voltage of up to 400 V and an operating temperature range of -25 to +60 °C, with cable glands at the bottom. The cabinet shall be wall-mounted and equipped with the following: Two three-pole circuit breakers, rated at 32 A with C characteristic , one three-pole circuit breaker rated at 63 A with C characteristic, Neutral (N) busbar, Protective earth (PE) busbar, Cable glands at the bottom All components shall be suitable for the intended AC photovoltaic installation and shall comply with the applicable IEC/EN standards and relevant safety requirements |  | piece |  | 1 |  |  |  |  |  |  |  |
| 7 |  | Power cable: PP 00-Y 5 × 6 mm², 0.6/1 kV |  | m |  | 6 |  |  |  |  |  |  |  |
| 8 |  | Power cable: PP 00 4 × 16 mm², 0.6/1 kV |  | m |  | 6 |  |  |  |  |  |  |  |
| 9 |  | Single-core cable: P/F-Y 1 × 16 mm² |  | m |  | 36 |  |  |  |  |  |  |  |
| 10 |  | Solar cable: PV1-F 1 × 6 mm², 1.8 kV |  | m |  | 300 |  |  |  |  |  |  |  |
| 11 |  | MC4 (+) connectors and pins |  | piece |  | 5 |  |  |  |  |  |  |  |
| 12 |  | MC4 (- ) connectors and pins |  | piece |  | 5 |  |  |  |  |  |  |  |
| 13 |  | Perforated cable tray with covers, dimensions 50 × 60 mm , |  | lump sum |  | 1 |  |  |  |  |  |  |  |
| 14 |  | Silicone, corrugated, self-extinguishing conduit, Ø 20–32 mm |  | m |  | 50 |  |  |  |  |  |  |  |
| 15 |  | Three-pole circuit breaker, rated at 63 A, with C characteristic |  | piece |  | 1 |  |  |  |  |  |  |  |
| 16 |  | Miscellaneous installation materials |  | lump sum |  | 1 |  |  |  |  |  |  |  |
| 17 |  | Complete technical documentation according to national law and legislation |  | piece |  | 1 |  |  |  |  |  |  |  |
| 18 |  | Installation and login to mobile application to record PV station parameters and training of two responsible employees |  | lump sum |  | 1 |  |  |  |  |  |  |  |
| 19 |  | Supply, transport and installation of a dual-outlet AC charging station for electric vehicles |  | piece |  | 2 |  |  |  |  |  |  |  |
| 20 |  | Additional equipment, occurred due to unexpected situation, from those planned in tech. document (max 10%) |  | lump sum |  | 1 |  |  |  |  |  |  |  |
| 21 |  | Information plaque with dimensions of 300 × 400 mm, manufactured from durable and weather-resistant material, such as metal with a protective coating, in accordance with the visibility and communication requirements of the Interreg VI-A Greece–North Macedonia 2021–2027 Programme. |  | piece |  | 2 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Total |  |  |  |

[DDP (D6elivered Duty Paid)][DAP (Delivered At Place)]  — Incoterms 2020 International Chamber of Commerce http://www.iccwbo.org/incoterms/